LITTLEBROTHER

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

PASSIVE RF INTELLIGENCE PLATFORM

Wi-Fi  ·  BLE  ·  Cellular  ·  IMSI Detection  ·  Evasion Automation

[ ARCHITECTURE & MODULE SPECIFICATION ]

v0.1.0-alpha  ·  2026

01  PROJECT OVERVIEW

LittleBrother is a cross-platform passive RF intelligence tool targeting Android (primary), iOS (reduced), Linux, macOS, and Windows. It collects, correlates, and analyzes Wi-Fi, Bluetooth LE, and cellular radio signals without transmitting, injecting, or jamming — operating entirely within stock OS APIs and legal passive-monitoring constraints.

The core threat model is civilian OPSEC: detecting surveillance infrastructure (IMSI catchers, rogue APs, persistent BLE trackers), building an offline intelligence baseline per location, and automating evasion responses when threats are confirmed.

▸ DESIGN PRINCIPLES

| PRINCIPLE | IMPLEMENTATION |
| --- | --- |
| Offline-first | All processing local; no telemetry; optional crowdsource sync |
| No root required | Stock Android APIs only on primary target |
| Passive only | No injection, no jamming, no spoofing — legal everywhere |
| Cross-platform UI | Flutter with per-platform capability gating |
| Persistent memory | SQLite threat DB; all observations stored and indexed |
| Modular pipeline | Scanners → Normalizer → Analyzer → Alert → UI |

▸ PLATFORM CAPABILITY MATRIX

| CAPABILITY | ANDROID | IOS | LINUX | MACOS/WIN |
| --- | --- | --- | --- | --- |
| Wi-Fi scan (passive) | ✓ Full | ✗ Locked | ✓ Full | ✓ Partial |
| BLE passive scan | ✓ Full | ✓ Limited | ✓ Full | ✓ Full |
| Cell tower data | ✓ Full | ✗ No API | ✗ No API | ✗ No API |
| GPS geotag | ✓ Full | ✓ Full | ✓ Full | ✓ Partial |
| Stingray detection | ✓ Full | ✗ No cell | ✗ No cell | ✗ No cell |
| OPSEC mode (RF kill) | ✓ Partial* | ✗ | ✓ nmcli | ✓ Partial |
| BLE tracker detection | ✓ Full | ✓ Limited | ✓ Full | ✓ Full |
| Intel DB review | ✓ Full | ✓ Full | ✓ Full | ✓ Full |

* Requires WRITE_SETTINGS permission; airplane mode toggle available on Android only

02  TECHNOLOGY STACK

▸ FRAMEWORK DECISION: FLUTTER

Flutter is chosen over React Native for this use case. The decisive factors: (1) BLE and cellular platform channel packages are more mature in the Flutter ecosystem (flutter_blue_plus, wifi_scan, telephony); (2) Flutter's compiled Dart runs closer to native speed for continuous signal processing loops; (3) the Canvas API gives full control over the radar HUD rendering without bridging overhead; (4) single codebase compiles to Android, iOS, Linux, macOS, and Windows without ejecting.

▸ CORE DEPENDENCIES

| PACKAGE | PURPOSE |
| --- | --- |
| flutter_blue_plus | BLE scanning — passive, no pairing required |
| wifi_scan | Wi-Fi AP enumeration with full scan results |
| telephony | Cell tower data (Android only): Cell ID, LAC, RSSI, type |
| geolocator | GPS position with accuracy metadata |
| sqflite | Local SQLite — threat DB, intel timeline, signal history |
| flutter_local_notifications | Foreground service + threat alerts |
| permission_handler | Unified runtime permission flow |
| http / dio | Optional: OpenCelliD sync, crowdsource push |
| fl_chart | RSSI time-series graphs per device |
| csv / share_plus | Session export (JSON, CSV) |

▸ PLATFORM CHANNELS (KOTLIN / ANDROID)

Several capabilities require native Kotlin code exposed via Flutter MethodChannel:

| CHANNEL | CAPABILITY |
| --- | --- |
| lb_radio/cell | getAllCellInfo(), PhysicalChannelConfig, ServiceState polling |
| lb_radio/wifi_ext | Scan throttle override, passive probe log (Android 10+) |
| lb_system/opsec | Airplane mode toggle (WRITE_SETTINGS), Wi-Fi/BT disable |
| lb_system/wake | Partial wakelock for background scan loop |

03  DATA ARCHITECTURE

▸ CORE DATA PIPELINE

All signals flow through a five-stage pipeline before reaching the UI or storage:

| STAGE | COMPONENT | RESPONSIBILITY |
| --- | --- | --- |
| 1 | Scanner | Platform-specific radio polling — Wi-Fi, BLE, Cell, GPS |
| 2 | Normalizer | Map raw OS results → unified LBSignal model; OUI lookup; distance calc |
| 3 | Analyzer | Anomaly scoring, stingray heuristics, tracker detection, baseline diff |
| 4 | AlertEngine | Rule evaluation → push notification, UI flash, OPSEC trigger |
| 5 | IntelStore | SQLite persistence — append observations, update baselines |

▸ SQLITE SCHEMA

DATABASE: littlebrother.db

TABLE: observations

| COLUMN | TYPE / NOTES |
| --- | --- |
| id | INTEGER PK AUTOINCREMENT |
| session_id | TEXT — UUID per scan session |
| signal_type | TEXT — wifi ¦ ble ¦ cell ¦ cell_neighbor |
| identifier | TEXT — BSSID / MAC / cell_id composite |
| display_name | TEXT — SSID / device name / carrier |
| rssi | INTEGER — dBm |
| distance_m | REAL — estimated meters (path loss model) |
| risk_score | INTEGER — 0–100 |
| lat | REAL — GPS latitude at scan time |
| lon | REAL — GPS longitude at scan time |
| metadata_json | TEXT — type-specific fields (see below) |
| ts | INTEGER — Unix ms timestamp |

TABLE: known_devices

| COLUMN | TYPE / NOTES |
| --- | --- |
| identifier | TEXT PK |
| signal_type | TEXT |
| display_name | TEXT |
| vendor | TEXT — OUI resolved |
| first_seen | INTEGER |
| last_seen | INTEGER |
| observation_count | INTEGER |
| threat_flag | INTEGER — 0=clean 1=watch 2=hostile |
| notes | TEXT |

TABLE: cell_baseline

| COLUMN | TYPE / NOTES |
| --- | --- |
| geohash | TEXT — 7-char geohash (≈ 150m cell) |
| cell_id | TEXT — MCC+MNC+LAC+CID composite |
| network_type | TEXT — LTE ¦ NR ¦ UMTS ¦ GSM |
| avg_rssi | REAL |
| first_seen | INTEGER |
| observation_count | INTEGER |
| opencellid_verified | INTEGER — 0/1 |

TABLE: threat_events

| COLUMN | TYPE / NOTES |
| --- | --- |
| id | INTEGER PK AUTOINCREMENT |
| threat_type | TEXT — stingray ¦ rogue_ap ¦ ble_tracker ¦ downgrade |
| severity | INTEGER — 1 (info) to 5 (critical) |
| identifier | TEXT — triggering device/cell |
| evidence_json | TEXT — heuristic scores that fired |
| lat | REAL |
| lon | REAL |
| ts | INTEGER |
| dismissed | INTEGER — 0/1 |

▸ METADATA JSON SCHEMAS BY SIGNAL TYPE

The metadata_json column in observations carries type-specific fields:

{ signal_type: "wifi" }

| FIELD | DESCRIPTION |
| --- | --- |
| frequency_mhz | Integer — e.g. 2437, 5180, 6135 |
| channel | Integer — derived from frequency |
| band | String — 2.4GHz ¦ 5GHz ¦ 6GHz |
| security | String — OPEN ¦ WEP ¦ WPA2 ¦ WPA3 ¦ ENT |
| capabilities | String — raw Android capabilities string |
| vendor | String — OUI-resolved manufacturer |
| channel_width_mhz | Integer — 20 ¦ 40 ¦ 80 ¦ 160 |

{ signal_type: "ble" }

| FIELD | DESCRIPTION |
| --- | --- |
| manufacturer_data | Hex string — raw manufacturer data bytes |
| manufacturer_id | Integer — Bluetooth SIG company ID |
| vendor | String — resolved from company ID |
| service_uuids | Array<String> — advertised service UUIDs |
| connectable | Boolean |
| advertising_interval_ms | Integer — estimated from scan timing |
| is_randomized_mac | Boolean — detected MAC rotation |

{ signal_type: "cell" }

| FIELD | DESCRIPTION |
| --- | --- |
| mcc | Integer — Mobile Country Code |
| mnc | Integer — Mobile Network Code |
| lac_tac | Integer — Location/Tracking Area Code |
| cell_id | Integer — Cell Identity (CID) |
| pci | Integer — Physical Cell ID (LTE/NR) |
| earfcn_arfcn | Integer — Frequency channel number |
| bandwidth_mhz | Integer — (Android 12+) |
| timing_advance | Integer — TA value, proxy for distance |
| rsrp | Integer — Reference Signal Received Power (LTE) |
| rsrq | Integer — Reference Signal Received Quality |
| sinr | Integer — Signal to Interference+Noise Ratio |
| is_serving | Boolean — serving vs neighbor cell |
| neighbor_count | Integer — number of neighbors reported |

04  MODULE SPECIFICATIONS

▸ MOD-01 · WI-FI SCANNER

| PROPERTY | SPECIFICATION |
| --- | --- |
| Scan interval | 10s active foreground / 30s background (Android throttle limit) |
| API | wifi_scan package → WifiScan.instance.startScan() + scanResults stream |
| Fields captured | SSID, BSSID, RSSI, frequency, capabilities, timestamp |
| OUI lookup | Embedded OUI table (IEEE, ~35k entries, updated at build time) |
| Distance model | Path loss: d = 10^((TxPower - RSSI) / (10 × n)), n=2.7 indoors |
| Risk scoring | See Section 5 — Analyzer spec |
| Desktop fallback | nmcli / iwlist subprocess on Linux; CoreWLAN on macOS |

▸ MOD-02 · BLE SCANNER

| PROPERTY | SPECIFICATION |
| --- | --- |
| Mode | Passive scan (SCAN_MODE_LOW_LATENCY in foreground, LOW_POWER background) |
| API | flutter_blue_plus — FlutterBluePlus.startScan(withServices: []) |
| Permissions | BLUETOOTH_SCAN (no BLUETOOTH_CONNECT needed for passive) |
| Fields captured | MAC, localName, RSSI, manufacturerData, serviceUuids, connectable |
| MAC rotation detect | Compare advertising patterns across MAC changes — same mfr data = same device |
| Tracker signatures | AirTag (0x004C + length 0x12), Tile (0xFEED), Samsung (0x0075) |
| Interval estimate | Track scan timestamps → infer advertising cadence |

▸ MOD-03 · CELLULAR SCANNER (ANDROID ONLY)

| PROPERTY | SPECIFICATION |
| --- | --- |
| API | Kotlin MethodChannel → TelephonyManager.getAllCellInfo() |
| Poll interval | 5s — cell data changes slowly but TA/type changes are critical |
| Cell types | CellInfoLte, CellInfoNr, CellInfoWcdma, CellInfoGsm, CellInfoCdma |
| LTE fields | CID, TAC, PCI, EARFCN, RSRP, RSRQ, SINR, TA, band (API 30+) |
| NR (5G) fields | NCI, TAC, PCI, ARFCN, SS-RSRP, SS-RSRQ, SS-SINR |
| Neighbor cells | Captured from getAllCellInfo() non-serving entries |
| ServiceState | Polled for network type transitions (triggers downgrade alert) |

▸ MOD-04 · GPS / GEOTAG

| PROPERTY | SPECIFICATION |
| --- | --- |
| API | geolocator — Geolocator.getPositionStream() |
| Accuracy | LocationAccuracy.high in foreground; reduced in background |
| Geohash | 7-char geohash computed per fix (≈150m precision for baseline cells) |
| Attachment | GPS fix stamped on every observation at scan time |
| Staleness | Fix older than 30s flagged; observations marked gps_stale |

▸ MOD-05 · INTEL STORE

| PROPERTY | SPECIFICATION |
| --- | --- |
| Engine | sqflite (SQLite) — local, zero-dependency, no cloud |
| Write mode | Batch insert per scan cycle — minimize WAL pressure |
| Indexes | identifier + ts; geohash; session_id; threat_flag |
| Retention | Configurable — default 90 days rolling; manual purge |
| Export | JSON (full schema) and CSV (flat observations) via share_plus |
| Import | JSON sessions from other devices — merge by identifier + ts |

▸ MOD-06 · OPSEC / EVASION AUTOMATION

| PROPERTY | SPECIFICATION |
| --- | --- |
| RF kill | Airplane mode via WRITE_SETTINGS (Android); Wi-Fi + BT disable fallback |
| Trigger | AlertEngine threat_event with severity >= configurable threshold |
| Sequence | 1. Alert UI flash 2. Push notification 3. RF kill (if enabled) 4. Log event |
| Recovery | Manual — user re-enables RF; app prompts with threat summary |
| OPSEC mode | Suppress device probe broadcasts: Wi-Fi scan off, BLE scan passive-only |
| Desktop | nmcli radio off on Linux; Airport -setairportpower off on macOS |

05  ANALYZER & THREAT DETECTION

The Analyzer runs after each normalizer pass. It evaluates incoming observations against historical baseline, known-bad lists, and heuristic rule sets. Each heuristic produces a 0–100 score; scores combine into a composite threat score that drives alert severity.

▸ THREAT-01 · IMSI CATCHER / STINGRAY DETECTION

Seven independent heuristics, each scored 0–100. Composite = weighted sum, capped at 100.

| HEURISTIC | WEIGHT | DETECTION LOGIC |
| --- | --- | --- |
| Network Downgrade | 30 | LTE/NR → GSM/UMTS transition observed in ServiceState stream. GSM = highest risk (unencrypted). Weight proportional to speed of downgrade. |
| Unknown Cell ID | 25 | Cell ID not present in local OpenCelliD tile for this geohash. Novel tower at known location. Score = 100 if completely unknown, 50 if seen <3 times. |
| Anomalous Signal Strength | 20 | RSRP/RSSI significantly above path-loss-modeled expected value for distance. Catcher broadcasts at elevated power. Threshold: +15 dB above baseline. |
| Missing Neighbor List | 10 | Real eNodeB always broadcasts neighbor cells. If serving cell reports 0 neighbors, score 80. <3 neighbors in urban area: score 40. |
| Timing Advance Anomaly | 10 | TA value inconsistent with GPS-estimated distance to tower (requires OpenCelliD coords). TA 0 = <78m; mismatch > 2 TA units flags. |
| Cell ID Instability | 3 | Serving cell ID changes more than 3× in 60s without significant GPS movement. Catcher reassigns frequently. |
| Pilot Pollution | 2 | RSRP vs RSRQ gap > 10 dB suggests abnormal interference profile. Not definitive alone. |

Alert threshold: Score ≥ 40 → WARNING (orange). Score ≥ 65 → THREAT (red). Score ≥ 85 → CRITICAL (red flash + OPSEC trigger if enabled).

▸ THREAT-02 · ROGUE ACCESS POINT DETECTION

| HEURISTIC | LOGIC |
| --- | --- |
| SSID collision | SSID matches known AP but BSSID differs — evil twin candidate |
| OUI mismatch | BSSID OUI is consumer device vendor (phone, laptop) not AP vendor |
| Open + popular SSID | Unencrypted network with SSID matching common public names |
| Signal anomaly | AP appearing at much higher RSSI than historical baseline for this location |
| Rapid BSSID churn | BSSID changes on same SSID across scan cycles |

▸ THREAT-03 · BLE TRACKER DETECTION

| HEURISTIC | LOGIC |
| --- | --- |
| Known tracker signature | Manufacturer data matches AirTag, Tile, Samsung SmartTag, Chipolo patterns |
| Persistent follower | Same MAC (or fingerprinted device) seen across 3+ geohashes over time |
| Randomized MAC persistence | Device rotates MAC but advertising pattern (mfr data, services) stays consistent |
| Anomalous advertising rate | BLE device advertising at <100ms interval — aggressive tracking mode |
| Unrecognized manufacturer | No OUI match, no service UUIDs — covert device |

▸ WI-FI RISK SCORING

| FACTOR | SCORE CONTRIBUTION |
| --- | --- |
| Open network (no auth) | +40 |
| WEP encryption | +35 (crackable) |
| WPA/WPA2-TKIP | +15 (deprecated cipher) |
| Hidden SSID | +10 |
| 2.4GHz + high RSSI (< -50 dBm) | +10 (proximity) |
| OUI = known hostile / public feed | +30 |
| SSID on local watchlist | +50 |
| WPA3 + strong signal | -10 (trusted baseline) |

06  UI ARCHITECTURE

▸ SCREEN MAP

| SCREEN | DESCRIPTION |
| --- | --- |
| RadarHUD | Primary screen — animated sweep, blips per signal, risk rings, live counts |
| SignalList | Tabbed list view: Wi-Fi ¦ BLE ¦ Cell ¦ Threats — sortable, filterable |
| DeviceDetail | Full observation history, RSSI graph, metadata, threat flag controls |
| IntelTimeline | Chronological session log — searchable, exportable |
| ThreatLog | All threat_events — severity filter, dismiss, evidence detail |
| CellMap | Map overlay — pinned observations, tower locations, threat markers |
| OPSECPanel | Evasion controls — RF kill toggle, alert thresholds, watchlist management |
| Settings | Scan intervals, permissions, DB management, crowdsource sync, export |

▸ RADAR HUD — RENDERING SPEC

Rendered on Flutter CustomPainter canvas at 60fps. No third-party chart library — direct canvas drawing.

| ELEMENT | RENDERING DETAIL |
| --- | --- |
| Background | Black fill; subtle radial gradient from center (dark blue tint) |
| Sweep arm | Rotating line from center, 0→360° at configurable RPM (default 4s/rev) |
| Sweep trail | Fading arc behind sweep arm — opacity decays over 90° of trail |
| Range rings | 3 concentric circles: ~10m, ~30m, ~100m (path-loss calibrated) |
| Grid lines | 8 radial spokes at 45° intervals; very low opacity |
| Signal blips | Dots placed at polar coords (angle=hash(id), radius=distance); size=RSSI strength |
| Blip color | Green=clean, Yellow=watch, Orange=warning, Red=threat, Cyan=cell tower |
| Blip decay | Blips fade over 3 sweep cycles if not refreshed by new scan |
| Threat flash | Full-canvas red pulse on CRITICAL threat event |
| Live counters | Corner overlays: AP count, BLE count, Cell ID, threat score |
| Compass rose | Subtle N/S/E/W labels; rotates with device orientation if sensor available |

▸ DESIGN LANGUAGE

| TOKEN | VALUE |
| --- | --- |
| Primary font | Courier New (monospace — tactical aesthetic, universal availability) |
| Background | #0A0A0A — near-black |
| Surface | #141414 — cards, panels |
| Accent | #3B82F6 — blue (borders, titles, active states) |
| Signal OK | #00FF88 — green |
| Signal warn | #FFD700 — gold |
| Threat | #FF4444 — red |
| Data text | #00D4FF — cyan (identifiers, values) |
| Body text | #C8C8C8 — light gray |
| Dim text | #6B6B6B — labels, secondary |

07  DEVELOPMENT ROADMAP

| PHASE | MODULE | DELIVERABLE |
| --- | --- | --- |
| P1 | Foundation | Flutter project scaffold, platform channels stub, SQLite schema, permission flow |
| P1 | Radar HUD | CustomPainter radar with animated sweep, static blip placement, live counter overlays |
| P2 | Wi-Fi Scanner | Live AP scan → normalizer → IntelStore; risk scoring; blips on radar |
| P2 | BLE Scanner | Passive BLE → normalizer → store; tracker signature matching; radar blips |
| P3 | Cell Scanner | Kotlin channel → TelephonyManager; full LTE/NR/GSM field capture; cell blips |
| P3 | Signal List | Tabbed list UI with sort/filter; DeviceDetail screen; RSSI history graph |
| P4 | Analyzer | Stingray heuristics engine; rogue AP; BLE tracker; composite scoring |
| P4 | Alert Engine | Threat events; push notifications; severity thresholds; OPSEC trigger |
| P5 | Intel Timeline | Session management; search; JSON/CSV export |
| P5 | OPSEC Panel | RF kill controls; watchlist management; evasion automation config |
| P6 | Cell Map | Map overlay; OpenCelliD integration; threat pin rendering |
| P6 | Crowdsource | Optional threat sync; anonymous submission; import threat feeds |
| P7 | iOS Port | Reduced-capability build; BLE + GPS only; UI parity |
| P7 | Desktop Port | Linux/macOS; nmcli/CoreWLAN subprocess; DB review and export focus |

Testing environment: Debian (N3XU5). Android target: Galaxy S25 Ultra (Android 15, API 35). Each phase produces a runnable build before the next phase begins.

08  LEGAL & OPERATIONAL BOUNDARIES

LittleBrother operates entirely within passive monitoring. The following constraints are architectural, not optional:

| WHAT WE DO | WHAT WE DO NOT |
| --- | --- |
| Observe radio signals already present in environment | Transmit, inject, or broadcast any signal |
| Read OS-provided scan results | Use monitor mode or raw 802.11 frame capture |
| Correlate against public tower databases | Spoof cell tower identity or IMSI |
| Alert user to suspicious patterns | Jam, interfere with, or disrupt any signal |
| Automate flight mode / RF kill on user device | Intercept communications content |
| Export intelligence for personal/research use | Deploy on devices without owner consent |

Active jamming and spoofing are federal crimes (18 U.S.C. § 1362, FCC Part 97). The "destruction" capability referenced in early planning is implemented exclusively as aggressive evasion automation — kill your own device's radio, log the event, alert the user. Nothing outward-facing.

IMSI catcher detection is a documented, legal field of security research. Tools like AIMSICD (Android IMSI-Catcher Detector, archived) and academic work from TU Berlin and SRLabs establish the legal and technical precedent for this class of application.